Specification

KS Track System

#101 Series Klick System Drapery Track

# Part 1-General

  - Summary

- Work of this section includes manual window treatment hardware as manufactured by Forest Group USA, Inc.

  - Submittals

- Product Data: Manufacturer’s descriptive literature shall be submitted, indicating materials, finishes and installation instructions for each type of window treatment specified.

  - Quality Assurance

- Products shall be manufactured to meet or exceed all quality standards as stipulated in the Quality Control Manual, issued by Forest Group USA, Inc.

  - Delivery, Storage and Handling

- Product shall be delivered to the project site in manufacturer’s original packaging.

- Product shall be handled and stored to prevent damage to materials, finishes and operating mechanisms.

  - Warranty

- Manufacturer shall provide that all system components are free of defects for a limited lifetime warranty.  This warranty does not apply to wear resulting from normal wear and tear or any damage loss caused by accidents; alterations; misuse, abuse or extraordinary use; improper installation; cleaning or other maintenance or acts of God.

- Warranties shall cease to be in effect if product has been moved from its original place of installation

- Warranties do not cover removal, installation, and replacement of shipping costs.

- Specific product warranty terms are available from Forest Group USA, Inc

# Part 2-Product

2.1 Manufacturer

- Acceptable product: #101 Series Klick System Drapery Track as manufactured by Forest Group, USA, Inc. 1500 Donn Drive, Cartersville, GA 30120

2.2 Equipment

- Light to medium duty hand drawn track system.

- Bendable to a 90 degree or less; curvable to a 4” radius.

- Can be used for ripplefold and accordiafold (pinch pleat).

- Maximum fabric weight 3 lbs. Per foot

2.3 Components

- Track:

- Shall be 6063-T5 chromated powder coated with factory applied patented dry lubricant.

- Shall have exterior dimensions of  ¾ “ width by ½ “ height.

- Parts:

- System shall include 3.5 carriers per foot and 1 bracket per ever 3 foot.

- Shall be original Forest Group, USA, Inc components.

# Part 3-Execution

3.1 Installation

A. Install window treatment system in accordance with Forest Group, USA, Inc

specifications, standards and procedures.  Installation contractor shall be responsible

for site measurements and suitability of mounting surface.

- Adequate clearance shall be provided to permit unencumbered operation of the

entire system.