GUIDE SPECIFICATIONS - WINDOW TREATMENT

Division 12 - Furnishings

Section 12552.01 - Manual Chain Operated Shades

1.0 	GENERAL

1.1 	RELATED SECTIONS:

1.1.1		Section 	06100 Rough Carpentry

1.1.2		Section		08520 Aluminum Windows

1.1.3		Section		08920 Glazed Aluminum Curtain Wall

1.1.4		Section		09250 Gypsum Board

1.1.5		Section		09510 Acoustic Ceilings

Products supplied by this section and installed by other trades: Specifier co-ordinate this work with designated trades in other sections of these project specifications.

1.1.6		Aluminum or steel shade pockets recessed into either gypsum or system ceilings.

1.1.7		Aluminum, ceiling pocket or housing trim profiles.

1.2 	SUMMARY:

1.2.1		Provide chain operated interior roller shade (single solar shade), or (single black out shade) or (dual shade) system, with related hardware to complete the installation to the following locations: Specifier provide scope of work.

Roller shades shall be mounted: Specifier select applicable position.

Between the vertical window framing, On the face of the window framing, To the underside of the ceiling or bulkhead, In a by this section provided metal pocket or housing recessed in the ceiling.

1.3 	QUALITY ASSURANCE:

1.3.1		All Bidders submitting Bids on the Work of this Section shall meet or exceed the quality of materials, components and assemblies specified herein.

Bidders who do not comply with these performance Specifications shall refrain from submitting a Bid.

1.3.2		Alternate Bids, Alternative Products shall be submitted to the Design Consultant 10 days prior to tender closing. Alternative Bids can only be submitted as an alternate Bid to the specified base Bid Product. Manufacturers that meet the performance criteria and are approved as an alternate by the Design Consultant shall be listed in an addendum.

1.3.3		Manufacturer shall have a minimum of Fifteen (15) years experience in the manufacture of specified shading system.

1.3.4		All work specified under this section supplied and installed entirely by one Subcontractor using his own forces.

1.3.5		Shades to be installed by a firm, with a minimum of Ten (10) years experience, specializing in the installation of shading systems.

1.3.6		Conform to applicable Building Code and Local Authority having jurisdiction and all other standards noted.

1.3.7		Install one complete operating sample with accessories on site. Review the installation before proceeding with the remainder of the work. Adjust sample installation to gain acceptance. Accepted work may form part of the final installation.

1.4 	SUBMITTALS AND TEST REPORTS:

1.4.1		Submit fully detailed drawings prepared in AutoCAD R14 format showing all components, finishes and perimeter construction conditions, installation, and all applicable dimensions according to Section 01300 - Submittals.

1.4.2		Submit duplicate samples of the specified fabric / shade cloth of each colour and texture; minimum size 8.5@ x 11@ (215mm x 280mm) for review.

1.4.3		Submit duplicate samples of the specified sections of fascia, closure, pocket, housing, trim, roller tube, hembar, operating hardware brackets and side channel for review.

1.4.4		Submit duplicate copies of compliance to the following test:

Fire :NFPA 701-89 Sm., NFPA 701-89 Lg. Flat, NFPA 701-96 TM#1.

Toxicity: UPITT.

Fungal Resistance: ASTM G 21.

Bacterial Resistance: ASTM G 22.

1.4.5		Submit duplicate copies of operating and maintenance instructions including, name and telephone number of local service company.

1.5	INSPECTION / PREPARATION:

1.5.1		Verify that all blocking and framing necessary to carry shade assembly hardware is a proper installation and secure.

1.5.2		Notify Owner in writing of any deficiencies in the work of other trades that would effect the window treatment system.

1.5.3		Make accurate measurements at the site before fabrication. Check layout of glazing framing sections, spans, and loading capabilities.

1.6 	DELIVERY, STORAGE AND HANDLING:

1.6.1		Do not deliver to site until areas to receive shades are completely finished, all walls and ceilings completed and painted.

1.6.2 	Deliver materials in original protective wrappings or containers, with manufacturers labels and sealed intact.

1.6.3		Handle and store materials according to manufacturers recommendations, protecting materials and finishes from damages, marring of finishes or soiling.

1.7 	WARRANTY:

1.7.1		Provide a limited manufacturers warranty from Date of Substantial Completion, covering the following period:

Shade hardware and operating chain: ten (10) years

Fabrics / shade cloth: ten (10) years

Aluminum and Steel coatings: ten (10) years

1.7.2 	Provide a limited installation warranty from Date of Substantial Completion, covering a period of one (1) year.

1.7.3		Submit standard manufacturers maintenance contract for review by Owner.

2.0 	PRODUCTS

2.1 	MANUFACTURER:

2.1.1		Nysan7 Shading Systems Ltd., 229 33 Street N.E. Calgary, Alberta, Canada, T2A 4Y6. Telephone: 403 569 4551, Facsimile: 403 569 4552, E-mail sales@nysan.com, Website: http://www.nysan.com.

Specifier please note when approving an alternate manufacturer please ensure that the performance criteria as set forth in these specifications (Quality Assurance 1.3.1 and 1.3.2) are met. This will ensure a fair price comparison and will not create a disadvantage for the specified manufacturer.

Alternative Bids can only be submitted as an alternate Bid to the specified base Bid Product. Manufacturers that meet the performance criteria and are approved as an alternate by the Design Consultant shall be listed in an addendum.

2.2 	MATERIALS:

2.2.1		Manual chain operator:

A.	Unitized premoulded construction, on 71.5 mm x 76 mm (2.875" x 3"), 12 gauge, AL@ shaped, coated steel mounting brackets, shipped completely assembled from factory.

- Operator assembly shall allow for continuous front or back-roll fascia across multiple shades (to a maximum length of 6.1m (20ft.)) without exposed fasteners.

- A single manual chain operator as designed by the specified manufacturer shall be capable of operating a shade band up to 5.2m2 (56 ft2) in total fabric area, 2.13m (7 ft) in width, or 2.44m (8 ft) in height, and shall be manufactured with a precise inertia brake mechanism capable of locking the shade panel or band at any point of travel.

Specifier please note: Should shade panel or band exceed the above dimensions, or if it is desired to drive more than one shade band with one operator, operating hardware should be specified as follows:

Chain operator type, Nysan7 SunriserTM, gear reduction operating hardware, manufactured with a precise inertia braking mechanism to stop shade at any desired point of travel.

D.	Chain operator assembly shall be set for travel length of shade, on-site without disassembly of hardware.

E.	Drive chain to be #10 stainless steel, tested strength of 41 kg (90lb.), to eliminate breakage of chain under normal usage.

- Shade roller tube shall be positively engaged to the drive mechanism through its internal profile configuration. Friction fitted engagement of the roller tube by the drive mechanism will not be acceptable.

G.	Shade roller tube shall be removable from brackets without hardware removal.

H.	Chain operator shall prohibit operation by pulling the hembar downward.

- All non-metal components shall be self-lubricating.

Specifier please note the following available options:

J.	Left hand, right hand or dual left and right operating systems.

- Offset drive operators available (chain falls only on one side of bracket). Offset operators shall be standard with blackout fabric shades.

2.2.2 	Roller tube assembly:

A.	Top roller tube of one piece extruded aluminum tube, with 10 micron thick clear anodized coating, at the manufacturers recommended engineered diameter and wall thickness for maximum allowable deflection of L/700;

Mill finish aluminum tubes will not be acceptable.

- The roller tube shall be extruded with provision made for mechanical engagement with the operator and drive assembly.

C.	The extrusion shall have various channels to accept fabric attachment spline. The spline and slot reinforces the tube and retains the fabric and operating system.

D.	The spline will be an extruded vinyl profile, welded to the fabric band or panel, such that removal and re-installation of the fabric bands or panels shall be without removing the roller tube and hardware. Fabric bands or panels must be replaceable on site.

Attachment of the fabric to the tube with double-sided adhesive tapes, adhesives, staples or rivets is not acceptable.

2.2.3		Hembars and Hembar Pockets: Specifier please select,

A.	Option 1: Custom shaped AFlat@ steel profile, 37 mm (1.5") high, wall thickness engineered to weight requirements and adapt to uneven surfaces, in welded hembar pocket with closed ends, to maintain bottom of shade fabric straight, and flat.

or

B.	Option 2: Ergonomic designed exposed extruded aluminum alloy 6063-T5, custom oval shaped profile 35 mm x 10 mm x 1.78 mm thick (1.377" x 0.394" x 0.070"), with matching end caps, pre-weighted, to maintain bottom of shade fabric straight and flat. Colour prefinished to match adjacent window framing or as selected by Consultant. Underside of hem bar available with schlegel light seal. Attached to fabric panel by welded fabric spline (as per section 2.2.2.d.).

or

C.	Option 3: Round shaped profile, 15 mm (0.625") diameter, wall thickness engineered to weight requirements, in welded hembar pocket with open ends. Finished with coloured PVC round end caps.

2.2.4		Fasteners:

A.	Non-corrosive to manufacturers recommendations.

2.3	FABRIC:

2.3.1		All fabric specifications and options may be found in the fabric section of this manual. This information may also be found in the Nysan7 fabric sample cards and on the Website at http://www.nysan.com.

2.4	OPTIONAL ACCESSORIES:

2.4.1		Aluminum Fascia: Specifier note: Fascia selection shall depend upon the type of fabric used, roll direction, and site conditions (site conditions may require use of custom fascia in a size other than those specified below. All other specifications shall remain applicable).

A.	Back / Regular Roll Solar Shade Fascia:

a.	Extruded aluminum alloy 6063-T5, prefinished, 78.5 mm x 52 mm x 1.6 mm wall thickness (3.09" x 2.047" x 0.063"), continuous custom designed profile to fit onto premoulded end mounting brackets without exposed fasteners or additional adhering materials. Colour prefinished to match adjacent window framing or as selected by Consultant.

- Fascia shall allow for continuous placement across multiple shades (to a maximum length of 6.1m (20ft.)) without exposed fasteners.

c.	Fascia shall conceal the mounting hardware, drive mechanism, roller tube, and all fabric rolled on the tube.

- Drive chain shall pass through punched openings in bottom of fascia.

B.	Back / Regular Roll Blackout Shade Fascia:

a.	Extruded aluminum alloy 6063-T5, prefinished, 105 mm x 45 mm x 1.6 mm wall thickness (4.13" x 1.77" x 0.063"), custom designed profile to fit onto premoulded end mounting brackets without exposed fasteners. Colour prefinished to match adjacent window framing or as selected by Consultant.

- Fascia shall allow for continuous placement across multiple shades (to a maximum length of 6.1m (20ft.)) without exposed fasteners.

- Fascia shall conceal the mounting hardware, drive mechanism, roller tube, and all fabric rolled on the tube.

- Fascia shall not fit snug against side channels (when specified) to prevent thermal shock to the glazing system.

Specifier please note that the manufacturer strongly recommends that snug fitted side channel and fascia systems be used only in situations where complete darkness is absolutely necessary (ie. Photograph development lab) and the possibility of thermal shock to the window system has been carefully investigated.

- e.	Drive chain shall pass through punched openings in bottom of fascia.

C.	Front / Reverse Roll Solar Shade Fascia:

a.	Extruded aluminum alloy 6063-T5, prefinished, 78.5 mm x 10 mm x 1.6 mm wall thickness (3.09" x 0.394" x 0.063"), custom designed closure to fit onto premoulded end mounting brackets without exposed fasteners. Colour prefinished to match adjacent window framing or as selected by Consultant.

- Reverse fascia shall allow for continuous placement across multiple shades (to a maximum length of 6.1m (20ft.)) without exposed fasteners.

- Reverse fascia shall conceal the mounting hardware and drive mechanism.

D.	Front / Reverse Roll Blackout Shade Fascia:

a.	Extruded aluminum alloy 6063-T5, prefinished, 105 mm x 10 mm x 1.6 mm wall thickness (4.13" x 0.394" x 0.063"), custom designed closure to fit onto premoulded end mounting brackets without exposed fasteners. Colour prefinished to match adjacent window framing or as selected by Consultant.

- b.	Reverse fascia shall allow for continuous placement across multiple shades (to a maximum length of 6.1m (20ft.)) without exposed fasteners.

- Reverse fascia shall conceal the mounting hardware, drive mechanism.

- Reverse fascia shall not fit snug against side channels (when specified) to prevent thermal shock to the glazing system.

Specifier please note that the manufacturer strongly recommends that snug fitted side channel and fascia systems be used only in situations where complete darkness is absolutely necessary (ie. Photo development lab) and the possibility of thermal shock to the window system has been carefully investigated.

2.4.2		Pockets, Closures and Channels: Specifier note: Pocket and closure selection shall depend upon the type of fabric used, roll direction, and site conditions (site conditions may require use of custom pockets and closures other than those specified below).

- Shade Pocket:

- Manufactured from 2.5 mm (0.098") thick aluminum or 1.5 mm (0.059") thick satincoat steel pocket engineered to meet site conditions. Design considerations include, but are not limited to, mounting conditions, wall/ceiling construction, type and quantity of shades in pocket, and shade length, as indicated on the drawings.

B.	Aluminum Closure:

- Extruded aluminum alloy, 6063-T5, prefinished, 82 mm wide x 1.5 mm thickness (3.23" x  0.059"). Colour shall be prefinished to match the adjacent window framing or as selected by the Consultant.

b.	Designed for attachment to the corresponding pocket and/or hardware using an extruded aluminum clip and return angle, both made of 6063-T5 alloy, without exposed fasteners.

C.	Room darkening side and bottom channel:

a.	Continuous extruded aluminum alloy, 6063-T5, U-shaped 51 mm x 22 mm x 1.5 mm thick  (2" x 0.875" x 0.0625") with interior channels for 6 mm (0.25") wide continuous schlegel light seals. Colour shall be prefinished to match the adjacent window framing or as selected by the Consultant.

- Custom manufactured for easy snap-on installation, allowing channel to snap into place without exposed fasteners; and to be removable without damage to the substrate.

2.5 	ALUMINUM FINISHES:

2.5.1		All exposed aluminum shall be clear anodized oxide finish according to AA-M12C22A31 or coloured to match window framing.

2.5.2		Unexposed aluminum unless otherwise specified: mill finish.

3.0 	EXECUTION

3.1	FABRICATION:

3.1.1		Shading system components manufactured and assembled allowing for custom installation techniques to suit project requirements.

3.1.2		Finished assemblies shall be, square, true to size and free from distortion, twist, or other defects that could affect their strength, operation or appearance. Factory applied finish shall be uniform, smooth and without blemishes.

3.1.3		The fabric shall be colourfast, retain its shape, not be affected by moisture or heat, and shall be non-flammable. Cut fabric to eliminate glare and reverberation from shining surfaces while maintaining exterior view. The top of the fabric is retained in recessed spline of the shade roller and the bottom of the fabric is retained by the selected hem.

Specifier please note: Battens will not be required in the majority of shades. In the event of unusually wide or long shades, contact the manufacturer at the location specified in section 2.1 for verification of any batten requirements.

3.2	INSTALLATION:

3.2.1		Install work by manufacturers skilled tradesmen and install in strict accordance with manufacturers recommendations.

3.2.2		All items installed, plumbed, squared, rigidly coupled and adequately anchored, maintaining uniformed clearances, accurate alignment levels, and parallel with the window plane to provide smooth operation without binding. Fabric shall not travel sideways more than 3 mm (0.125") in either direction.

3.2.3		The solar screen and / or blackout fabric shall be pre-measured and manufactured off-site.

3.3	ADJUSTING AND CLEANING:

3.3.1		Adjust shades and operating components as required to ensure smooth and trouble free operation without binding.

3.3.2		Adjust shade and shadecloth to hang flat without buckling or distortion.

3.3.2		Clean shades and exposed components.

3.3.4		Replace work which cannot be satisfactorily repaired, adjusted, or cleaned.

END