Project Name:

SECTION 12492

Horizontal Louver Blinds

# Part 1 - General

# DESCRIPTION

- 2" Horizontal Composite Faux Wood Blinds

  - SUBMITTALS

- Manufacturer’s complete CSI 3- part specification sheet.

- Product Sample: Submit working hand sample or mock up blind as required.

- Color Sample: Submit two 6" samples of slat indicating color and dimensions.

  - DELIVERY, STORAGE AND HANDLING

- Product to be delivered in manufacturer’s original packaging.

- Products to be handled and stored to prevent damage to materials, finishes and operating mechanisms. Store in a clean, dry area, laid flat to prevent sagging and twisting of packaging.

  - EXTRA STOCK

- Describe extra attic stock as required:

.

# Part 2 – Products

  - ACCEPTABLE MANUFACTURERS

- Springs Window Fashions or approved equivalent.

- Substitutions Request: Submit for approval under provisions of section 01600.

  - HORIZONTAL BLINDS

- Product: Graber Traditions 2” Composite Faux Wood Blinds

- Color

Color Name:

Color Number:

- Headrail: Shall be 1 1/2" high by 2 1/4" wide, U-shaped, with 1/8” light blocking lip, on the bottom center line, made of phosphate treated steel, finished with a polyester baked enamel paint finish and shall measure .022" thick.

- Bottomrail: Shall be a blend of PVC polymers 7/8” high by 2” wide in a trapezoid shape and shall coordinate with slats.

- Slats:  Slats shall be a blend of PVC polymers  2” wide and 1/8” thick that meet the requirement of  NFPA 701 Test Method 1-2004 edition.  Slat edges are straight cut.

Options:

NoHoles Slats

- Cord tilter shall be a snap-in component incorporating a worm and pulley of low friction thermoplastic and a gear of nylon. Tilt cords shall be secured to pulley and treated with wood tassels at tilt end.

Select One:

Cord tilter (standard)

Wood wand tilter

Ring tilter

- Cord lock shall be metal of a snap-in design incorporating a floating shaft-type locking pin.  Cord lock shall incorporate a crash proof safety-feature that will lock blind automatically upon release of cord.  End of lift cords will be treated with wood tassels.

Options:

Ring pull provides a single plated steel ring in lieu of tassels with a nominal 4" cord length.

- Braided ladder shall be made of 100% high tenacity polyester incorporating two extra strength rungs per ladder to support slats. Ladder will provide 44mm distance between slats.

Select One:

Braided Ladder (standard)

Cloth Tape

Cloth Tape Color Name:

Cloth Tape Color Number:

  - Fabrication

  - Prior to fabrication, verify actual opening dimensions by on-site measurement. Calculate blind dimensions to fit within specified tolerances.

  - Fabricate blinds to fill openings from head to sill and jamb-to-jamb. Locate blind divisions at mullions.

  - Fabricate blinds to fill all exterior window openings except at doors, door                                sidelights and transoms unless noted.

## Part 3 – Execution

  - Inspection

A.  Verify that the area in which the blinds are to be installed is free of    conditions that interfere with blind installation and operations. Begin blind installation only when unsatisfactory conditions have been corrected.

  - Installation

  - Install blinds in accordance with manufacturer’s instructions except as otherwise specified herein.

  - Install blinds with adequate clearance to permit smooth operation of the blinds.

  - Set tilt and lift controls. Demonstrate blinds to be in smooth, uniform working order.

  - Maintenance and Cleaning

  - Maintain and clean blinds in accordance with manufacturer’s instructions.

### End of Section